Project report

crazyflie demonstration @ LNF22

Florian Huber

Institute of Networked and Embedded Systems

University of Klagenfurt

Table of contents

1.	Project overview	3

1.1	Used hardware	3

1.2	Used software	3

1.3	System overview	4

2	Workflow for the demo	5

2.1	Configuration	5

2.2	Execution of the demo	7

2.3	Review flown paths	9

# Project overview

The core part of this project was the development of a demonstration with the motion capture system “Optitrack” and the drone “crazyflie 2.1” of Bitcraze. This demo is shown at the “Lange Nacht der Forschung 2022” (LNF22) on the 20.05.2022.

## Used hardware

Host system:

- 8x Optitrack Flex13 Cameras

- 2x Optitrack OptiHub 2

- 1x Host PC (Win10)

Client system

- 1x Bitcraze crazyflie2.1

- 1x Bitcraze crazyradio PA Dongle

- 1x Client PC (Ubuntu 20.04)

- 1x Wand

## Used software

Host system:

- Motive Body 2.3.1

- MATLAB R2021b

Client system:

- crazyswarm			https://github.com/USC-ACTLab/crazyswarm

- crazyflie client			https://github.com/bitcraze/crazyflie-clients-python

- ROS Noetic Ninjemys		http://wiki.ros.org/noetic

- Python 3.8

## System overview

Figure 1 (overview of crazyswarm)

# Workflow for the demo

## Configuration

Host PC

- Start Motive

- Switch to the “Stream” Layout

Figure 2

- Check the streaming configuration

VRPN- and Trackd Streaming must be switch off, such as Asset Markers, Rigid Bodies and Skeltons.

Figure 3

Client PC

- Open a Terminal

- To check the crazyflies (eg. Battery, connection etc.) open the chooser.py

source crazyswarm/ros_ws/devel/setup.bash

cd crazyswarm/ros_ws/src/crazyswarm/scripts

python3 chooser.py

Figure 4

- Go back to root directory and open the crazyswarm server

(!! crazyflie and Host PC must be turned on and configured correctly at this point !!)

cd

roslaunch crazyswarm hover_swarm.launch

Figure 5

Figure 6

## Execution of the demo

- Open a new terminal to execute the python script of the demo

source crazyswarm/ros_ws/devel/setup.bash

cd crazyswarm/ros_ws/src/crazyswarm/scripts

[python3 LNF22_crazyflieDemo.py]

or

[./LNF22_crazyflieDemo.py]

- Then the script is executed and the type of dem must be chosen.

Figure 7

- To make a hover test (start, hover, land):	0

- To fly an 8 forwards and backward:		1

- To follow the wand in real-time:		2

Before starting the time to follow must be inserted and afterwards press any Button to start following.

Figure 8

Figure 9

- To mark 4 waypoints and fly through them afterwards:		3

Figure 10

- To draw a path with the wand and fly along it afterwards:		4

Before starting the time, in which the path shold be sampled, ,must be inserted.

Also to start sampling and to start the drone a key must be pressed.

Figure 11

## Review flown paths

To view the flown pathes or how well the drone follows the wand etc., a logfile must be created, exported from Motive and loaded into MATLAB.

Save tracking data:

Before starting the python script on the client, the tracking must be captured.

To do this Write a Takename and start capturing.

Figure 12

When the flight is finished, stop the take and switch to the Edit Layout.

Figure 13

Export tracking data:

Then select the markers in the 3D representation and go to “Export Tracking Data” and save the log with a unique name and the preferences shown in Figure 15 to “C:\Useres\PervasiveComputing\Desktop\LNF22\Logs”.

Figure 14

Figure 15

Review the tracking data in MATLAB:

If you only want to view the data of a simple flight of the drone (e.g., Flying a 8)

Visualization_DroneOnly.m		(Change name of log to exported file)

Figure 16

Figure 17

If you want to view the data of a wand and a drone

Visualization_WandDrone.m		(Change name of log to exported file)

Figure 18

Figure 19